STATIC TIMING ANALYSIS

# 16-Liberty-Model STA Assessment

Escrow Release Controller | SKY130 HD | OpenSTA 2.4.0

# 1. Executive summary

The setup WNS ranges from -5.86 ns to +7.26 ns. The worst reported setup result is -5.86 ns at sky130_fd_sc_hd__ss_n40C_1v28, indicating a setup violation under the current constraints. Worst hold slack is positive in all 16 rows, ranging from +0.30 ns to +1.69 ns. All results remain REVIEW.

TNS (total negative slack) was not captured in the supplied summary/log output. It is therefore marked NOT CAPTURED—not zero. The WNS graph is included; a numeric TNS graph requires rerunning the sweep with a TNS reporting command.

# 2. Methodology

OpenSTA 2.4.0.

Clock period: 10 ns.

Input delay: 2 ns; output delay: 2 ns.

Propagated clocks enabled.

Routed SPEF from the existing physical-design run.

The same routed SPEF was reused for all 16 Liberty models.

This is a preliminary multi-model timing assessment, not a signoff-validated PVT matrix. Timing constraints have not been independently validated for signoff.

# 3. Setup WNS by Liberty model

Figure 1. Setup WNS by model. Negative values indicate setup violations. TNS is not plotted because it was not captured.

# 4. Results

| Liberty model | Setup WNS (ns) | Hold worst slack (ns) | Status |
| --- | --- | --- | --- |
| ff_100C_1v65 | +7.05 | +0.39 | REVIEW |
| ff_100C_1v95 | +7.26 | +0.31 | REVIEW |
| ff_n40C_1v56 | +6.79 | +0.45 | REVIEW |
| ff_n40C_1v65 | +6.95 | +0.40 | REVIEW |
| ff_n40C_1v76 | +7.09 | +0.35 | REVIEW |
| ff_n40C_1v95 | +7.25 | +0.30 | REVIEW |
| ss_100C_1v40 | +3.91 | +1.43 | REVIEW |
| ss_100C_1v60 | +5.29 | +0.99 | REVIEW |
| ss_n40C_1v28 | -5.86 | +1.48 | REVIEW |
| ss_n40C_1v35 | -0.86 | +1.63 | REVIEW |
| ss_n40C_1v40 | +1.31 | +1.69 | REVIEW |
| ss_n40C_1v44 | +2.59 | +1.56 | REVIEW |
| ss_n40C_1v60 | +4.92 | +1.03 | REVIEW |
| ss_n40C_1v76 | +5.84 | +0.75 | REVIEW |
| tt_025C_1v80 | +6.72 | +0.47 | REVIEW |
| tt_100C_1v80 | +6.74 | +0.48 | REVIEW |

# 5. Critical setup path

At sky130_fd_sc_hd__ss_n40C_1v28, the critical path starts at _116_ (timeout_count_q[0]) and ends at _120_ (timeout_count_q[4]). Data arrival is 15.71 ns, data required time is 9.84 ns, and setup slack is -5.86 ns. The largest incremental delay in the supplied path listing is 5.67 ns at _060_ (sky130_fd_sc_hd__or4b_4). This identifies an investigation point but does not prove that one RTL expression alone causes the violation.

# 6. Excluded model variants

Excluded from the 16-model sweep:

sky130_fd_sc_hd__ff_n40C_1v95_ccsnoise.lib

sky130_fd_sc_hd__ss_n40C_1v60_ccsnoise.lib

These are excluded CCS-noise model variants, not additional PVT corners.

# 7. Warnings and limitations

OpenSTA reported sky130_fd_sc_hd__tapvpwrvgnd_1 unavailable in the linked netlist and created a black box for PHY_28.

Liberty warning: default_fanout_load = 0.0.

Same routed SPEF reused across all models.

Timing constraints need independent validation.

TNS was not captured in the supplied output.

# 8. Required follow-up for WNS/TNS graph

Update the per-corner OpenSTA Tcl script to emit total negative slack for setup, rerun the 16-model sweep, and regenerate the CSV, graph, and report. Do not substitute WNS for TNS or assume TNS is zero.